- Tilt sensor

- Air pressure-altitude

- Internal temperature

- Battery Voltage

- GPS Position

- Ascent and Descent Speed- pitot tube

- Camera feed #not necessarily real time

The probe telemetry shall include

- altitude

- air pressure

- temperature

- battery voltage

- probe tilt angles

- air speed

- command echo

- GPS coordinates that include latitude, longitude, altitude and number of satellites tracked.

The displays shall be designed

- using larger fonts (14 point minimum)

- bold plot traces and axes

- a dark text on light background theme

The ground system shall count the number of received packets.

The Cansat telemetry packet format to be transmitted at one (1) Hz is as follows:

TEAM_ID, MISSION_TIME, PACKET_COUNT, MODE, STATE, ALTITUDE, AIR_SPEED, HS_DEPLOYED, PC_DEPLOYED, TEMPERATURE, VOLTAGE, PRESSURE, GPS_TIME, GPS_ALTITUDE, GPS_LATITUDE, GPS_LONGITUDE, GPS_SATS, TILT_X, TILT_Y, ROT_Z, CMD_ECHO [,,OPTIONAL_DATA]

The telemetry data fields are to be formatted as follows:

1. TEAM_ID is the assigned four digit team identification number. E.g., imaginary team ‘1000’.

2. MISSION_TIME is UTC time in format hh:mm:ss, where hh is hours, mm is minutes, and ss is seconds. E.g., ‘13:14:02’ indicates 1:14:02 PM.

3. PACKET_COUNT is the total count of transmitted packets since turn on, which is to be reset to zero by command when the Cansat is installed in the rocket on the launch pad at the beginning of the mission and maintained through processor reset.

4. MODE = ‘F’ for flight mode and ‘S’ for simulation mode.

5. STATE is the operating state of the software. (e.g., LAUNCH_WAIT, ASCENT, 14 ROCKET_SEPARATION, DESCENT, HS_RELEASE, LANDED, etc.). Teams may define their own states. This should be a human readable description as the judges will review it after the launch in the .csv files.

6. ALTITUDE is the altitude in units of meters and must be relative to ground level at the launch site. The resolution must be 0.1 meters.

7.  AIR_SPEED is the air speed in meters per second measured with the pitot tube during both ascent and descent

8. HS_DEPLOYED = ‘P’ indicates the heat shield is deployed, ‘N’ otherwise.

9. PC_DEPLOYED = ‘C’ indicates the parachute is deployed (at 100 m), ‘N’ otherwise.

10. TEMPERATURE is the temperature in degrees Celsius with a resolution of 0.1 degrees.

11. PRESSURE is the air pressure of the sensor used. Value must be in kPa with a resolution of 0.1 kPa.

12. VOLTAGE is the voltage of the Cansat power bus with a resolution of 0.1 volts.

13. GPS_TIME is the time from the GPS receiver. The time must be reported in UTC and have a resolution of a second.

14. GPS_ALTITUDE is the altitude from the GPS receiver in meters above mean sea level with a resolution of 0.1 meters.

15. GPS_LATITUDE is the latitude from the GPS receiver in decimal degrees with a resolution of 0.0001 degrees North.

16. GPS_LONGITUDE is the longitude from the GPS receiver in decimal degrees with a resolution of 0.0001 degrees West.

17. GPS_SATS is the number of GPS satellites being tracked by the GPS receiver. This must be an integer.

18. TILT_X, TILT_Y are the angles of the Cansat X and Y axes in degrees, with a resolution of 0.01 degrees, where zero degrees is defined as when the axes are perpendicular to the Z axis which is defined as towards the center of gravity of the Earth.

19. ROT_Z is the rotation rate of the Cansat in degrees per second with a resolution of 0.1 degrees per second.

20. CMD_ECHO is the text of the last command received and processed by the Cansat. For example, CXON or SP101325. See the command section for details of command formats. Do not include commas characters. 15

21. [,,OPTIONAL_DATA] are zero or more additional fields the team considers important following two commas, which indicates a blank field. This data must follow the same format rules (including use of comma characters ‘,’) to facilitate review of the CSV files by the judges after the mission.

It is suggested that teams make use of onboard data storage as backup in case of radio failure. Only the transmitted telemetry is examined and scored on flight day; however, the backup data can be used when preparing the Post Flight Review presentation.

Commands

The payload shall receive and process the following commands from the Ground Station:

CX - Payload Telemetry On/Off Command

CMD,<TEAM_ID>,CX,<ON_OFF>

Where:

1. CMD and CX are static text.

2. <TEAM ID> is the assigned team identification.

3. <ON_OFF> is the string ‘ON’ to activate the payload telemetry transmissions

and ‘OFF’ to turn off the transmissions.

Example: The command CMD,1000,CX,ON activates payload telemetry transmission,

assuming the team id is 1000.

ST - Set Time

CMD,<TEAM_ID>,ST,<UTC_TIME>|GPS

Where:

4. CMD and ST are static text.

5. <TEAM ID> is the assigned team identification.

6. <UTC_TIME>|GPS is UTC time in the format hh:mm:ss or ‘GPS’ which sets the

flight software time to the current time read from the GPS module.

Example: The command CMD,1000,ST,13:35:59 sets the mission time to the value

given and the command CMD,1000,ST,GPS sets the time to the current GPS time.

Note: It is recommended that the time be set directly from the Ground System time, in

UTC, or from the GPS rather than being typed into the command manually.

SIM - Simulation Mode Control Command

CMD,<TEAM_ID>,SIM,<MODE>

Where:

1. CMD and SIM are static text.

2. <TEAM_ID> is the assigned team identification.

3. <MODE> is the string ‘ENABLE’ to enable the simulation mode, ‘ACTIVATE’ to

activate the simulation mode, or ‘DISABLE’ which both disables and deactivates

the simulation mode.

Example: Both the CMD,1000,SIM,ENABLE and CMD,1000,SIM,ACTIVATE

commands are required to begin simulation mode.

Note: It is advised that care be taken to not allow mixing of simulated and actual

barometric altitude data. This caused at least one failure in the recent launches.

SIMP - Simulated Pressure Data (to be used in Simulation Mode only)

CMD,<TEAM ID>,SIMP,<PRESSURE>

Where:

1. CMD and SIMP are static text.

2. <TEAM ID> is the assigned team identification.

3. <PRESSURE> is the simulated atmospheric pressure data in units of pascals

with a resolution of one Pascal.

Example: CMD,1000,SIMP,101325 provides a simulated pressure reading to the

payload (101325 Pascals = approximately sea level). Note: this command is to be

used only in simulation mode.

Note: Pressure values in the SIMP profile are not calibrated to be relative to the

launch site altitude, but are absolute altitudes above sea level.

CAL - Calibrate Altitude to Zero

CMD,<TEAM ID>,CAL

The CAL command is to be sent when the Cansat is installed on the launch pad and

causes the flight software to calibrate the telemetered altitude to 0 meters.

Note: This command also can be used by the flight software to reset and enable

processor reset recovery algorithms. (Note: more than three teams failed to complete

the mission last year because of processor resets during the launch; so, be prepared.)

BCN - Control Audio Beacon

CMD,<TEAM ID>,BCN,ON|OFF

Where:

1. CMD and BCN are static text.

2. <TEAM ID> is the assigned team identification.

3. <ON|OFF> are static strings “ON” or “OFF” that control the audio beacon

The BCN command allows the audio beacon, which is normally activated only upon

landing, to be activated and deactivated for testing and inspection and the flight

readiness review.

OPTIONAL - Optional Commands

The team may implement additional commands that are useful for testing or controlling

the Cansat. For example: it is a good idea to implement commands to activate

mechanisms and the audible beacon for testing and demonstration purposes.

Note: commanding during flight is a risky operation as radio links and precise timing

are unreliable; so, autonomous Cansat flight software operation should be the primary

mechanism for mission success.

Simulation Mode does not affect sensor data other than the barometric pressure sensor. In

Simulation Mode, the telemetered pressure sensor data should reflect the commanded

simulation values, not the actual sensor readings.

The purpose of Simulation Mode is to provide a mechanism for testing the Cansat as well as

for demonstrating the Cansat over the full mission profile in the event an actual launch cannot

take place (due to weather or pandemic).

The team’s Ground Station shall have the capability to read a barometric pressure data file

that simulates the mission profile and transmit the values via commands to the Cansat at a

rate of one per second. The pressure profile data file will be provided by the competition

judges, and may vary between teams.

Simulation Mode Concept of Operation

For testing, pre-flight demonstration, and in a contingency where launch operations are not

possible, a profile of barometric pressure data that represents the expected flight profile can

be transmitted by the team’s Ground System at a rate of one sample per second. Upon

receipt, the flight software will substitute the received value for the actual pressure sensor

reading.

The probe telemetry operates the same in FLIGHT and SIMULATION mode. So, the values,

other than the pressure and altitude (calculated from the pressure values) will be actual

sensor readings (e.g., actual battery, temperature, and GPS).

Because accidental initiation of SIMULATION mode can endanger the flight mission, two

commands, SIM ENABLE and SIM ACTIVATE, must be received by the flight software before

Simulation mode is activated.

After activation, the flight software monitors the radio link for barometric pressure sensor

commands (SIMP) from the Ground Station and uses the received values as if they were

actual barometric pressure readings in the calculation of altitude, determination software

state, and when to release the probe.

| GCS Overview | A simple context diagram showing major components |  |  |  |  | 2 |
| --- | --- | --- | --- | --- | --- | --- |
| GCS Design | Should have a diagram of the ground station, including components and how they connect |  |  |  |  | 2 |
| GCS Antenna Trade & Selection |  |  |  |  |  |  |
| Trade (show at least 2) | Should include antenna patterns |  |  |  |  | 2 |
| Discuss Antenna Mounting Design | Handheld or table top |  |  |  |  | 2 |
| Selection | Indicate selected design and reasons for selection. |  |  |  |  | 2 |
| GCS Software | Should include telemetry display prototypes, commercial off the shelf software packages used, real time plotting software design, how the calibration command for the barometric sensor and row/pitch angles will be transmitted and verified, telemetry data recording and media presentation to judges, .csv telemetry file creation for judges. |  |  |  |  | 2 |